For combination cells (depending on the number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (pF) | Fall Cap (pF) | Average Cap (pF) |
| --- | --- | --- | --- |
| A | 0.0028979 | 0.00292195 | 0.00292195 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A:

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02440 | 0.01896 | 0.03477 |
| 10 fF | 0.68230 | 0.06873 | 0.07696 |
| 100 fF | 0.59246 | 0.59248 | 0.59306 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A:

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.22510 | 0.01793 | 0.32600 |
| 10 fF | 0.74120 | 0.07377 | 0.08190 |
| 100 fF | 0.64161 | 0.64165 | 0.64250 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A:

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.04929 | 0.06676 | 0.14594 |
| 10 fF | 0.08926 | 0.10664 | 0.19889 |
| 100 fF | 0.44732 | 0.46460 | 0.55730 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A:

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.04688 | 0.06883 | 0.15840 |
| 10 fF | 0.09671 | 0.11781 | 0.21900 |
| 100 fF | 0.53255 | 0.55408 | 0.65790 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (A) | Power (nW) |
| --- | --- |
| 0 | 0.1163 |
| 1 | 0.1163 |

- Dynamic Power Table:

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin A

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 339.62 | 361.63 | 364.24 |
| 10 fF | 1965.93 | 1972.5 | 2160.98 |
| 100 fF | 18111.4 | 18106.5 | 17994 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin A

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 197.13 | 212.47 | 247.94 |
| 10 fF | 170.06 | 173.89 | 243.33 |
| 100 fF | 171.46 | 170.74 | 173.19 |