For Sequential cells, the following characterizations have to be performed and filled.

- Input pin capacitances:

| Input Pins | Rise Cap (pF) | Fall Cap (pF) | Average Cap (pF) |
| --- | --- | --- | --- |
| D | 0.00216453 | 0.00217269 | 0.00216861 |
| CLK | 0.00520604 | 0.00485335 | 0.0050296 |

- Set-up Time Table:

(i) Rise Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0.110 | 0.800 |
| 1000 ps | 0.002 | 0.002 |

(ii) Fall Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0.150 | 0.600 |
| 1000 ps | 0.002 | 0.002 |

- Hold Time Table:

(i) Rise Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0* | 0* |
| 1000 ps | 0* | 0* |

(ii) Fall Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0* | 0* |
| 1000 ps | 0* | 0* |

*By theory we know that schematic we used for DFF have 0 hold time

- Transition Time Table (for Q output, CLK will have minimum slew of 10 ps): (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.03119 | 0.03119 | 0.03264 |
| 10 fF | 0.04830 | 0.04830 | 0.04766 |
| 100 fF | 0.20622 | 0.20622 | 0.20620 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.03081 | 0.03081 | 0.03043 |
| 10 fF | 0.05773 | 0.05773 | 0.05663 |
| 100 fF | 0.33051 | 0.33051 | 0.33057 |

- CLK-to-Q Delay Time Table: (delay between clock transition and data transition. Use 50% of CLK to 50% of output to simulate propagation delay).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.25495 | 0.25495 | 0.25438 |
| 10 fF | 0.28025 | 0.28025 | 0.28014 |
| 100 fF | 0.41035 | 0.41035 | 0.41039 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.20015 | 0.20015 | 0.20030 |
| 10 fF | 0.23393 | 0.23393 | 0.23404 |
| 100 fF | 0.45925 | 0.45925 | 0.45943 |

- Static Power (all possible input combinations of CLK and D).

| Condition (CLK, D) | Power (nW) |
| --- | --- |
| 00 | 98870 |
| 01 | 98870 |
| 10 | 144800 |
| 11 | 144800 |

- Dynamic Power Table: (CLK will have minimum slew of 10 ps)

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 11461.50 | 11460.52 | 14456.67 |
| 10 fF | 220.91 | 224.18 | 308.96 |
| 100 fF | 2286.14 | 2688.61 | 3197.86 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF |  |  |  |
| 10 fF |  |  |  |
| 100 fF |  |  |  |