For combination cells (depending on the number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (pF) | Fall Cap (pF) | Average Cap (pF) |
| --- | --- | --- | --- |
| A | 0.01073 | 0.01083 | 0.01078 |
| B | 0.00839 | 0.00845 | 0.00842 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (B at VDD)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02094 | 0.01883 | 0.03477 |
| 10 fF | 0.03612 | 0.03717 | 0.04912 |
| 100 fF | 0.22777 | 0.22765 | 0.22870 |

Related pin B: (A at GND)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02029 | 0.02809 | 0.10130 |
| 10 fF | 0.03864 | 0.04276 | 0.14210 |
| 100 fF | 0.21828 | 0.21828 | 0.30120 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (B at VDD)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02267 | 0.02175 | 0.03552 |
| 10 fF | 0.04251 | 0.04381 | 0.05315 |
| 100 fF | 0.26650 | 0.26640 | 0.26806 |

Related pin B: (A at GND)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.01821 | 0.02248 | 0.07928 |
| 10 fF | 0.04319 | 0.04417 | 0.11218 |
| 100 fF | 0.26647 | 0.26634 | 0.30660 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A: (B at VDD)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.04151 | 0.05802 | 0.11516 |
| 10 fF | 0.05613 | 0.07296 | 0.13614 |
| 100 fF | 0.18818 | 0.20473 | 0.27070 |

Related pin B: (A at GND)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.01925 | 0.03972 | 0.11010 |
| 10 fF | 0.03306 | 0.05474 | 0.15920 |
| 100 fF | 0.15788 | 0.17933 | 0.39340 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A: (B at VDD)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.04408 | 0.06324 | 0.14056 |
| 10 fF | 0.06349 | 0.08293 | 0.16595 |
| 100 fF | 0.23609 | 0.25549 | 0.34224 |

Related pin B: (A at GND)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02856 | 0.04312 | 0.07971 |
| 10 fF | 0.04699 | 0.06291 | 0.13671 |
| 100 fF | 0.21889 | 0.23657 | 0.39665 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (ABC) | Power (nW) |
| --- | --- |
| 00 | 0.01236 |
| 01 | 1.76453 |
| 10 | 0.01236 |
| 11 | 1.76453 |

- Dynamic Power Table:

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 797.18 | 915.94 | 1063.46 |
| 10 fF | 2354.95 | 2395.21 | 3001.17 |
| 100 fF | 18408.1 | 18386.8 | 18388.2 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 865.84 | 1277.3 | 1378.28 |
| 10 fF | 2517.16 | 3109.42 | 3025 |
| 100 fF | 18722.8 | 18720.5 | 19283.5 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 828.08 | 883.78 | 1073.41 |
| 10 fF | 828.58 | 840.64 | 1109.53 |
| 100 fF | 864.94 | 842.24 | 1401.38 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 777.39 | 772.36 | 1018.86 |
| 10 fF | 814.6 | 836.23 | 895.69 |
| 100 fF | 851.82 | 841.93 | 760.89 |